# 1. Introduction

This assignment involves designing and configuring an IP addressing scheme for a medium-sized company network. The network was built and simulated using Cisco Packet Tracer and consists of two ISR4331 routers, two Catalyst 2960-24TT switches, and two PC workstations forming two separate Local Area Networks (LANs) connected via a WAN link.

IP addressing is a fundamental aspect of network design. Without a structured and well-planned addressing scheme, devices on a network would be unable to communicate reliably. This report documents the full process: selecting a base network, applying Variable Length Subnet Masking (VLSM) to create efficient subnets, assigning IP addresses to all devices, and verifying the design.

The base network selected for this design is 192.168.10.0/24 — a Class C private address commonly used in enterprise and educational environments. VLSM is applied to divide this block into three subnets of varying sizes, matching the actual host requirements of each network segment and minimising IP address wastage.

# 2. Objectives

The objectives of this assignment are:

- To design a complete IP addressing scheme for a medium-sized company network.

- To apply VLSM subnetting to efficiently divide the base network 192.168.10.0/24.

- To assign appropriate IP addresses, subnet masks, and default gateways to all devices.

- To simulate and test the network in Cisco Packet Tracer.

- To document the full IP allocation in a clear and structured table.

- To demonstrate understanding of routing between subnets and WAN link configuration.

# 3. Network Topology

The network topology built in Cisco Packet Tracer consists of two department LANs connected through two routers. The topology includes the following devices:

- Router1 (ISR4331) — connects to BIT1102 Switch (Gi0/0) and Router2 via WAN link (Gi0/1).

- Router2 (ISR4331) — connects to BIT1204 Switch (Gi0/0) and Router1 via WAN link (Gi0/1).

- BIT1102 Switch (Catalyst 2960-24TT) — provides Layer-2 switching for the BIT1102 LAN.

- BIT1204 Switch (Catalyst 2960-24TT) — provides Layer-2 switching for the BIT1204 LAN.

- PC BIT 1102 — end-user workstation on the BIT1102 department LAN.

- PC BIT 1204 — end-user workstation on the BIT1204 department LAN.

Figure 1: Cisco Packet Tracer Network Topology – Two LANs Connected via ISR4331 Routers

As shown in Figure 1, each LAN segment follows a simple star topology where all devices connect through a central switch. The two routers are connected via a point-to-point WAN link, enabling inter-LAN communication. The green link indicators confirm that all physical connections are active in the simulation.

# 4. IP Addressing and Subnetting Concepts

## 4.1 IP Address Structure

An IPv4 address is a 32-bit number expressed in dotted-decimal notation, divided into four octets (e.g., 192.168.10.0). Every IP address consists of two parts: the network portion, which identifies the subnet, and the host portion, which identifies individual devices within that subnet. The subnet mask determines where this boundary lies.

Private IP address ranges defined by RFC 1918 include: 10.0.0.0/8 (Class A), 172.16.0.0/12 (Class B), and 192.168.0.0/16 (Class C). These addresses are used in internal networks and are not routable on the public internet. The base network 192.168.10.0/24 used in this assignment falls within the Class C private range.

## 4.2 Variable Length Subnet Masking (VLSM)

VLSM is a subnetting technique that allows the creation of subnets of different sizes from the same base network. Unlike fixed-length subnetting, VLSM allocates address space based on the actual number of hosts required in each subnet. This reduces IP address wastage significantly and is considered best practice in modern network design.

The VLSM process follows a simple rule: always subnet from the largest requirement to the smallest. The formula used to calculate the required block size is:

2ⁿ  ≥  (Number of Required Hosts  +  2)

The +2 accounts for the network address and broadcast address that are reserved in every subnet and cannot be assigned to hosts. Once n is determined, the prefix length is calculated as 32 − n.

# 5. Base Network and Subnet Design

## 5.1 Base Network

The selected base network is 192.168.10.0/24, which provides 256 total addresses (254 usable for hosts). This network must be subdivided to serve three segments: the BIT1102 LAN (requires ~50 hosts), the BIT1204 LAN (requires ~30 hosts), and the WAN link between the two routers (requires exactly 2 hosts).

## 5.2 VLSM Step-by-Step Process

Subnets are carved from the base network in order from largest to smallest:

- Step 1 — BIT1102 LAN needs 50 hosts: 2⁷ = 128 ≥ 52  →  /25 block. Assign 192.168.10.0/25.

- Step 2 — BIT1204 LAN needs 30 hosts: 2⁶ = 64 ≥ 32  →  /26 block. Assign 192.168.10.128/26.

- Step 3 — WAN Link needs 2 hosts: 2² = 4 ≥ 4  →  /30 block. Assign 192.168.10.192/30.

- Remaining block 192.168.10.196 – 192.168.10.255 is reserved for future network expansion.

## 5.3 Subnet Breakdown Table

| Subnet Name | Network Address | Subnet Mask | CIDR | Hosts Needed | Usable Hosts | Usable Range | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BIT1102 LAN | 192.168.10.0 | 255.255.255.128 | /25 | 50 | 126 | 192.168.10.1 – .126 | 192.168.10.127 |
| BIT1204 LAN | 192.168.10.128 | 255.255.255.192 | /26 | 30 | 62 | 192.168.10.129 – .190 | 192.168.10.191 |
| WAN Link R1–R2 | 192.168.10.192 | 255.255.255.252 | /30 | 2 | 2 | 192.168.10.193 – .194 | 192.168.10.195 |
| Future Reserve | 192.168.10.196 | — | — | — | — | 192.168.10.196 – .255 | — |

Table 1: VLSM Subnet Breakdown for 192.168.10.0/24

# 6. Detailed Subnetting Calculations

This section provides the full calculation for each of the three active subnets derived from the base network 192.168.10.0/24.

## 6.1 Subnet 1 – BIT1102 LAN (/25)

| Parameter | Value |
| --- | --- |
| Hosts Required | 50 |
| Formula Applied | 2^7 = 128 ≥ 52 (50 + 2) |
| Host Bits (n) | 7 |
| Prefix / Mask | /25 → 255.255.255.128 |
| Network Address | 192.168.10.0 |
| Usable Range | 192.168.10.1 – 192.168.10.126 |
| Broadcast | 192.168.10.127 |
| Usable Hosts | 126 |

Table 2: BIT1102 LAN Subnet Calculation

## 6.2 Subnet 2 – BIT1204 LAN (/26)

| Parameter | Value |
| --- | --- |
| Hosts Required | 30 |
| Formula Applied | 2^6 = 64 ≥ 32 (30 + 2) |
| Host Bits (n) | 6 |
| Prefix / Mask | /26 → 255.255.255.192 |
| Network Address | 192.168.10.128 |
| Usable Range | 192.168.10.129 – 192.168.10.190 |
| Broadcast | 192.168.10.191 |
| Usable Hosts | 62 |

Table 3: BIT1204 LAN Subnet Calculation

## 6.3 Subnet 3 – WAN Link Router1 to Router2 (/30)

A /30 subnet is the standard choice for point-to-point WAN links. It provides exactly 2 usable host addresses, one assigned to each router interface, with minimal address waste.

| Parameter | Value |
| --- | --- |
| Hosts Required | 2 (one per router) |
| Formula Applied | 2^2 = 4 ≥ 4 (2 + 2) |
| Host Bits (n) | 2 |
| Prefix / Mask | /30 → 255.255.255.252 |
| Network Address | 192.168.10.192 |
| Usable Range | 192.168.10.193 – 192.168.10.194 |
| Broadcast | 192.168.10.195 |
| Usable Hosts | 2 |

Table 4: WAN Link Subnet Calculation

## 6.4 Address Space Utilisation

| Subnet | Addresses Allocated | Addresses Remaining |
| --- | --- | --- |
| BIT1102 LAN (/25) | 128 | 0 |
| BIT1204 LAN (/26) | 64 | 0 |
| WAN Link (/30) | 4 | 2 (net + bcast) |
| Reserved | 60 | — |
| Total from /24 | 256 | — |

Table 5: Address Space Utilisation Summary

# 7. Device IP Allocation Table

The following table documents the complete IP address assignment for every interface and device in the network. This serves as the definitive configuration reference for the network administrator.

| Device / Interface | Subnet | IP Address | Subnet Mask | Default Gateway | Role |
| --- | --- | --- | --- | --- | --- |
| Router1 Gi0/0 | BIT1102 LAN | 192.168.10.1 | 255.255.255.128 | — | Default Gateway – BIT1102 |
| Router1 Gi0/1 | WAN Link | 192.168.10.193 | 255.255.255.252 | — | WAN Interface |
| Router2 Gi0/0 | BIT1204 LAN | 192.168.10.129 | 255.255.255.192 | — | Default Gateway – BIT1204 |
| Router2 Gi0/1 | WAN Link | 192.168.10.194 | 255.255.255.252 | — | WAN Interface |
| BIT1102 Switch | BIT1102 LAN | 192.168.10.2 | 255.255.255.128 | 192.168.10.1 | Layer-2 Switch (Mgmt) |
| BIT1204 Switch | BIT1204 LAN | 192.168.10.130 | 255.255.255.192 | 192.168.10.129 | Layer-2 Switch (Mgmt) |
| PC BIT 1102 | BIT1102 LAN | 192.168.10.10 | 255.255.255.128 | 192.168.10.1 | End-user Workstation |
| PC BIT 1204 | BIT1204 LAN | 192.168.10.140 | 255.255.255.192 | 192.168.10.129 | End-user Workstation |

Table 6: Full Device IP Address Allocation Table

## 7.1 Allocation Notes

- Router interfaces use the first usable address in each subnet — a best practice that makes gateways easy to remember.

- Switch management IPs use the second address in each subnet, enabling remote Telnet or SSH administration.

- Workstation IPs are drawn from the middle of the usable range, leaving space for future devices.

- Dashes (—) in the Default Gateway column for routers indicate they route via their routing table, not a gateway.

- All addresses are statically configured — no DHCP server is used in this design.

# 8. Network Verification

After configuring all devices, the network must be tested to confirm correct operation. The following Cisco IOS commands are used for verification:

- ping [destination IP] — tests basic IP connectivity between two devices.

- traceroute [destination IP] — shows the hop-by-hop path packets take through the network.

- show ip route — displays the router's routing table to confirm all subnets are reachable.

- show ip interface brief — summarises all interfaces, their IP addresses, and operational status.

Key tests to perform in Cisco Packet Tracer:

- PC BIT 1102  →  ping  192.168.10.1  (Router1 Gi0/0, same subnet — should succeed immediately).

- PC BIT 1102  →  ping  192.168.10.140  (PC BIT 1204, cross-subnet — tests end-to-end routing).

- Router1  →  ping  192.168.10.194  (Router2 Gi0/1, WAN link test).

- Both routers: run show ip route to verify all three subnets appear in the routing table.

# 9. Common Issues and Troubleshooting

During network configuration, various issues can arise. The table below documents the most common problems, their causes, and the recommended solutions.

| Issue | Possible Cause | Recommended Solution |
| --- | --- | --- |
| IP address conflict | Two devices share same IP | Reassign using the allocation table — every IP must be unique |
| Wrong subnet mask | Host cannot reach its gateway | Match the mask to the subnet (e.g., /25 = 255.255.255.128) |
| Incorrect default gateway | Local ping works; remote fails | Set gateway to the router interface IP of that subnet |
| WAN link mis-config | Routers cannot ping each other | Verify /30 IPs on both Gi0/1 interfaces and check routing |
| Missing static route | Packets dropped at router | Add static routes or enable a dynamic protocol (RIP / OSPF) |

Table 7: Common Network Issues and Troubleshooting Guide

When troubleshooting, always follow the OSI model from the bottom up — verify physical links first, then data link (switch ports), then network layer (IP addresses, masks, gateways), and finally routing configuration. This systematic approach quickly isolates the source of any problem.

# 10. Discussion

The subnetting exercise carried out in this assignment demonstrates the practical value of VLSM in real-world network design. By allocating a /25 to BIT1102 LAN, a /26 to BIT1204 LAN, and a /30 to the WAN link, the entire 192.168.10.0/24 block is used efficiently — only 60 addresses are set aside as a reserve for future growth, and nothing is wasted.

If fixed-size subnetting had been used instead (for example, all /26 subnets), the WAN link alone would consume 64 addresses when only 2 are needed — wasting 60. Across a large enterprise with hundreds of WAN links, this approach would consume address space rapidly. VLSM solves this problem elegantly.

One area for improvement in the current design is redundancy. If the WAN link or either router fails, both LANs will be completely isolated. A production network would benefit from redundant links and a dynamic routing protocol such as OSPF, which automatically re-routes traffic around failures. The addition of VLANs on the switches would also improve security by isolating traffic between departments at Layer 2.

Despite these limitations, the design successfully meets all assignment requirements and reflects best practices in IP address planning for a small-to-medium enterprise environment.

# 11. Conclusion

This assignment successfully designed and implemented a complete IP addressing scheme for a medium-sized company network using Cisco Packet Tracer. Applying VLSM to the base network 192.168.10.0/24, three subnets were created — a /25 for BIT1102 LAN, a /26 for BIT1204 LAN, and a /30 for the WAN link — each sized to match actual host requirements with minimal address wastage.

All eight device interfaces were assigned unique, correctly-scoped IP addresses, documented in a detailed allocation table. The subnetting calculations follow standard VLSM methodology, and the network topology was verified in Cisco Packet Tracer. A troubleshooting guide and verification plan further demonstrate practical understanding of network administration principles.

IP addressing and subnetting are critical skills for any network administrator, and this assignment provided valuable hands-on experience in applying these concepts within a realistic simulated environment. The knowledge and techniques used here form the foundation for more advanced topics such as dynamic routing, VLAN segmentation, and network security.

# 12. References

- Cisco Systems, Inc. (2023). Cisco Packet Tracer User Guide. Cisco Networking Academy. [Online] Available at: https://www.netacad.com

- Forouzan, B. A. (2013). Data Communications and Networking. 5th ed. McGraw-Hill Education.

- Lammle, T. (2020). CompTIA Network+ Study Guide: Exam N10-008. 5th ed. Sybex / Wiley.

- Odom, W. (2022). CCNA 200-301 Official Cert Guide, Volume 1. Cisco Press.

- RFC 1918 (1996). Address Allocation for Private Internets. IETF. [Online] Available at: https://www.rfc-editor.org/rfc/rfc1918

- Tanenbaum, A. S. and Wetherall, D. J. (2011). Computer Networks. 5th ed. Pearson Prentice Hall.

- Limkokwing University of Creative Technology (2026). DCOMP 114 – Network Administration Course Notes. Faculty of ICT, Sierra Leone Campus.

TABLE OF CONTENTS

Introduction…………………………………………………………………………………………………………………..

Objectives……………………………………………………………………………………………………………………..

Network Topology………………………………………………………………………………………………………….

IP Addressing and Subnetting Concepts……………………………………………………………………………

IP Address Structure………………………………………………………………………………………………………

Variable Length Subnet Masking (VLSM)…………………………………………………………………………..

Base Network and Subnet Design…………………………………………………………………………………….

- Base Network………………………………………………………………………………………………………

- VLSM Step-by Step Process……………………………………………………………………………………

- Subnet Breakdown Table………………………………………………………………………………………

Detailed Subnetting Calculations……………………………………………………………………………………..

- Subnet 1 – BIT1102 LAN (/25)……………………………………………………………………………..

- Subnet 2 – BIT1204 LAN (/26)………………………………………………………………………………

- Subnet 3 – WAN Link Router1 to Router2 (/30)………………………………………………………

- Address Space Utilisation………………………………………………………………………………………

Device IP Allocation Table……………………………………………………………………………………………….

Allocation Notes…………………………………………………………………………………………………………….

Network Verification………………………………………………………………………………………………………

Common Issues and Troubleshooting………………………………………………………………………………

Discussion…………………………………………………………………………………………………………………….

Conclusion…………………………………………………………………………………………………………………….

References……………………………………………………………………………………………………………………