Table of Contents

- Introduction
2.1 Assignment Objective
2.2 Overview of Network Administration

- Assignment 1: IP Addressing and Subnetting
3.1 Company Network Design
3.2 Network Requirements
3.3 IP Addressing Scheme
3.4 Subnetting Design Using VLSM
3.5 IT Department Subnet
3.6 Administration Department Subnet
3.7 Finance Department Subnet
3.8 Server Department Subnet
3.9 Subnet Allocation Table
3.10 Device IP Assignment Table
3.11 Network Topology Diagram
3.12 Subnetting Visualization

- Assignment 2: Router Configuration and Routing Protocols
4.1 Network Topology Overview
4.2 Router A Configuration
4.3 Router B Configuration
4.4 Router C Configuration
4.5 Static Routing Configuration
4.6 RIP Routing Configuration
4.7 OSPF Routing Configuration
4.8 Verification Commands
4.9 Connectivity Testing
4.10 Routing Table Verification
4.11 Troubleshooting Procedures

- Conclusion

- References

- Appendices
9.1 Cisco Packet Tracer Screenshots
9.2 CLI Configuration Outputs
9.3 Additional Network

IP Addressing and Subnetting

Objective

The objective of this assignment is to design and configure an IP addressing scheme for a medium-sized company. This involves subnetting a network into smaller segments, assigning IP addresses to network devices, calculating subnet masks, and documenting all allocations in a structured table.

Company Network Design

A medium-sized company network is divided into four departments:

- IT Department – 100 Hosts

- Administration Department – 50 Hosts

- Finance Department – 25 Hosts

- Server Department – 10 Hosts

The network address used for this design is:

192.168.10.0/24

Subnetting Design Using VLSM

Variable Length Subnet Masking (VLSM) is used to efficiently allocate IP addresses according to host requirements.

1. IT Department

- Required Hosts: 100

- Subnet Mask: 255.255.255.128

- CIDR Notation: /25

- Network Address: 192.168.10.0

- Usable Host Range: 192.168.10.1 – 192.168.10.126

- Broadcast Address: 192.168.10.127

Image Description

Figure 1: Network diagram showing the IT Department subnet connected to Router A. The image should display multiple computers connected through a switch to the router interface using subnet 192.168.10.0/25.

2. Administration Department

- Required Hosts: 50

- Subnet Mask: 255.255.255.192

- CIDR Notation: /26

- Network Address: 192.168.10.128

- Usable Host Range: 192.168.10.129 – 192.168.10.190

- Broadcast Address: 192.168.10.191

Image Description

Figure 2: Diagram illustrating the Administration subnet connected to a switch and router. The image should show PCs grouped under the Administration department with subnet 192.168.10.128/26.

3. Finance Department

- Required Hosts: 25

- Subnet Mask: 255.255.255.224

- CIDR Notation: /27

- Network Address: 192.168.10.192

- Usable Host Range: 192.168.10.193 – 192.168.10.222

- Broadcast Address: 192.168.10.223

Image Description

Figure 3: Network topology displaying the Finance Department connected to a router through a switch. The diagram should indicate subnet 192.168.10.192/27.

4. Server Department

- Required Hosts: 10

- Subnet Mask: 255.255.255.240

- CIDR Notation: /28

- Network Address: 192.168.10.224

- Usable Host Range: 192.168.10.225 – 192.168.10.238

- Broadcast Address: 192.168.10.239

Image Description

Figure 4: Server room network diagram showing file servers and web servers connected to the router using subnet 192.168.10.224/28.

Subnet Allocation Table

| Department | Network Address | Subnet Mask | Host Range | Broadcast Address |
| --- | --- | --- | --- | --- |
| IT | 192.168.10.0/25 | 255.255.255.128 | 192.168.10.1–126 | 192.168.10.127 |
| Administration | 192.168.10.128/26 | 255.255.255.192 | 192.168.10.129–190 | 192.168.10.191 |
| Finance | 192.168.10.192/27 | 255.255.255.224 | 192.168.10.193–222 | 192.168.10.223 |
| Servers | 192.168.10.224/28 | 255.255.255.240 | 192.168.10.225–238 | 192.168.10.239 |

Device IP Address Assignment

| Device | Department | IP Address |
| --- | --- | --- |
| Router Interface IT | IT | 192.168.10.1 |
| Switch IT | IT | 192.168.10.2 |
| Workstation 1 | IT | 192.168.10.10 |
| Router Interface Admin | Administration | 192.168.10.129 |
| Switch Admin | Administration | 192.168.10.130 |
| Admin PC | Administration | 192.168.10.135 |
| Router Interface Finance | Finance | 192.168.10.193 |
| Switch Finance | Finance | 192.168.10.194 |
| Finance PC | Finance | 192.168.10.200 |
| Server Gateway | Servers | 192.168.10.225 |
| File Server | Servers | 192.168.10.226 |
| Web Server | Servers | 192.168.10.227Image Description |

Figure 5: Screenshot of a completed IP allocation table or spreadsheet display assigned IP addresses for routers, switches, PCs, and servers.

Router Configuration and Routing Protocols

Objective

The objective of this assignment is to configure routers using static routes and dynamic routing protocols such as RIP and OSPF. Verification commands are used to test connectivity and troubleshoot network issues.

Network Topology

Three routers are connected to simulate inter-network communication.

- Router A

- Router B

- Router C

Router Configuration

Router A Configuration

enable

configure terminal

interface g0/0

Ip address 192.168.10.1 255.255.255.128

No shutdown

interface s0/0/0

Ip address 10.0.0.1 255.255.255.252

No shutdown

Router B Configuration

enable

configure terminal

interface s0/0/0

ip address 10.0.0.2 255.255.255.252

No shutdown

interface s0/0/1

Ip address 10.0.0.5 255.255.255.252

No shutdown

Image Description

Figure 8: Router B command-line interface showing serial interface configuration.

Router C Configuration

enable

configure terminal

interface s0/0/1

Ip address 10.0.0.6 255.255.255.252

No shutdown

interface g0/0

Ip address 192.168.20.1 255.255.255.0

No shutdown

Image Description

Figure 9: Router C CLI screenshot displaying IP configuration and enabled interfaces.

Static Routing Configuration

Router A

Ip route 192.168.20.0 255.255.255.0 10.0.0.2

Router C

Ip route 192.168.10.0 255.255.255.128 10.0.0.5

Image Description

Figure 10: Screenshot showing static route configuration commands entered the router terminal.

RIP Routing Configuration

Router A

router rip

version 2

network 192.168.10.0

network 10.0.0.0

no auto-summary

Router B

router rip

version 2

network 10.0.0.0

no auto-summary

Router C

router rip

version 2

network 192.168.20.0

network 10.0.0.0

no auto-summary

Image Description

Figure 11: .

OSPF Routing Configuration

Router A

router ospf 1

network 192.168.10.0 0.0.0.127 area 0

network 10.0.0.0 0.0.0.3 area 0

Router B

router ospf 1

network 10.0.0.0 0.0.0.255 area 0

Router C

router ospf 1

network 192.168.20.0 0.0.0.255 area 0

network 10.0.0.4 0.0.0.3 area 0

Image Description

Figure 12: Screenshot displaying OSPF configuration and routing process ID setup on routers.

Verification Commands

To verify network connectivity, the following commands are used:

ping 192.168.20.1

traceroute 192.168.20.1

show ip route

show ip protocols

Image Description

Figure 13: Screenshot showing successful ping replies and routing table verification.

Troubleshooting Routing Issues

Problem 1: No Connectivity

Solution

show ip interface brief

Check interface status and ensure interfaces are enabled.

Problem 2: Missing Route

Solution

show ip route

Add missing routes if networks are unreachable.

Problem 3: Routing Loop

Solution

- Verify routing tables.

- Remove duplicate or incorrect routes.

- Use OSPF for better routing efficiency.

Image Description

Figure 14: Screenshot showing troubleshooting commands and corrected routing table.

Conclusion

This report demonstrated how to design and configure a subnetted IP addressing scheme for a medium-sized company. It also explained router configuration, static routing, dynamic routing using RIP and OSPF, verification techniques, and troubleshooting methods.

The subnetting design efficiently allocated IP addresses to each department, while routing protocols ensured communication between networks. The configuration commands and verification steps proved successful network connectivity.

References

- Cisco Networking Academy Materials

- Computer Networking Fundamentals

- Cisco Packet Tracer Documentation

- Network Administration Course Notes

- RFC Standards for IP Addressing and Routing Protocols